Karnataka Wind Power Plants

Complete Dataset & Feature Reference for ML Generation Forecasting

Total State Installed Capacity: ~7,351 MW  |  Rank: #1 in India (FY 2024-25)  |  Reference: KREDL / MNRE / CEA

# 1. Karnataka Wind Power — State Overview

Karnataka is India's #1 wind energy state as of FY 2024-25, with a cumulative installed wind capacity of 7,351 MW, having added a record 1,331.48 MW in FY25 alone. The wind resource is concentrated in the northern and central districts — Vijayapura, Gadag, Chitradurga, Koppal, Ballari, Davangere, and Belagavi — where the Deccan Plateau's elevation creates consistent wind flows, especially during the Southwest Monsoon (June–September).

| # | Plant / Zone | District | Developer | Capacity (MW) | Turbine OEM | Hub Ht (m) | Comm. Period | Grid Connection |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Gadag Wind Farm | Gadag | ReNew Power / Sterlite | 302 | Siemens Gamesa / Suzlon | 120 | 2021-23 | KPTCL 220kV |
| 2 | Chitradurga Wind Cluster | Chitradurga | Greenko / Mytrah | ~350 | Siemens Gamesa | 100-120 | 2011-2022 | KPTCL 220kV |
| 3 | Vijayanagara (Hanumanthappa) | Vijayanagara | Sri Kumaraswamy / Suzlon | 25.2 | Suzlon S120 2.1MW | 120 | 2024 | GESCOM |
| 4 | Kanakagiribad Wind Farm | Ballari / Vijayanagara | Sembcorp Green Infra | 180.6 | Suzlon S120 2.1MW | 120 | 2024 | KPTCL 220kV |
| 5 | Bagalkot Wind Farm | Bagalkot | Sembcorp Green Infra | ~50 | Suzlon 2.1MW | 120 | 2024 | GESCOM |
| 6 | Vijayapura Wind Cluster | Vijayapura | Various (Mytrah, Greenko) | ~1060 | Siemens Gamesa / Enercon | 80-120 | 2010-2023 | KPTCL 220kV |
| 7 | Koppal Wind Zone | Koppal | Jindal Renewables (JSP) | 702+ | Suzlon S144 3.15MW | 140 | 2024-25 | KPTCL 220kV |
| 8 | Davangere Wind Farm | Davangere | Greenko / Adani | ~200 | Siemens Gamesa | 100-120 | 2015-2022 | KPTCL |
| 9 | Haveri Wind Farm | Haveri | Multiple IPPs | ~150 | Enercon / WEG | 100 | 2014-2020 | HESCOM |
| 10 | Bidar / Kalaburagi Zone | Kalaburagi | Adani Green / CLP | ~120 | Siemens Gamesa | 100 | 2017-2022 | GESCOM |
| 11 | Raichur Wind Farm | Raichur | Greenko / KPCL | ~80 | Suzlon / WEG | 80-100 | 2008-2018 | GESCOM |
| 12 | Bellary (Kudligi) Zone | Ballari | Mytrah Energy / KPCL | ~180 | Gamesa / Enercon | 80-100 | 2012-2020 | GESCOM |
| 13 | Chikkaballapur Wind | Chikkaballapur | Vibrant Energy (Hybrid) | ~100 | Siemens Gamesa | 120 | 2023-24 | BESCOM |
| 14 | Tumkur (Sira) Wind Farm | Tumakuru | ReNew Power | ~75 | Siemens Gamesa | 100 | 2016-2020 | BESCOM |
| 15 | Belgaum (Chikkodi) Cluster | Belagavi | Multiple (Suzlon, WEG) | ~200 | Suzlon / WEG | 80-120 | 2005-2021 | HESCOM |

Note: Capacity figures marked '~' are approximate aggregates from multiple commissioned projects within the zone per KREDL district-wise data. The Vijayapura and Koppal zones are the largest contributors. Data sources: KREDL.karnataka.gov.in, CEA Resource Adequacy Plan 2024, MNRE.

# 2. Complete Feature Set for Wind Generation Forecasting Model

The table below covers every input variable required to train a robust wind power forecasting model — from raw meteorological drivers and plant-static parameters to operational SCADA data, temporal features, NWP forecast inputs, and derived/engineered features.

| Feature / Variable | Unit | Resolution | Data Source | Why It Matters / How Used |
| --- | --- | --- | --- | --- |
| METEOROLOGICAL (Primary Drivers) |  |  |  |  |
| Wind Speed at Hub Height (V_hub) | m/s | 10-min / 1-hr | NWP (ERA5, MERRA2, GFS) | Most critical — direct input to power curve (P ∝ V³) |
| Wind Direction at Hub Height | degrees (0-360°) | 10-min / 1-hr | NWP / SCADA anemometer | Determines yaw alignment, wake losses |
| Wind Speed at 10m, 50m, 100m | m/s | 1-hr | ERA5 reanalysis / met mast | Vertical wind shear profile calculation |
| Air Temperature | °C | 1-hr | IMD / NWP | Air density correction: ρ = P/(RT) |
| Atmospheric Pressure | hPa | 1-hr | IMD / ERA5 | Air density correction; affects power output |
| Relative Humidity | % | 1-hr | IMD / ERA5 | Affects air density; icing risk |
| Turbulence Intensity (TI) | % (σV/V_mean) | 10-min | SCADA / Lidar | Fatigue, output variability |
| Wind Shear Exponent (α) | dimensionless | 1-hr | Computed from multi-height speeds | Height extrapolation: V2 = V1 × (H2/H1)^α |
| Wind Veer | °/m | 1-hr | NWP / Lidar | Directional change with height; affects rotor loads |
| PLANT / TURBINE STATIC PARAMETERS |  |  |  |  |
| Installed Capacity (Prated) | MW per turbine | Static | OEM datasheet / KREDL | Max generation ceiling; input to normalization |
| Number of Turbines (N) | count | Static | KREDL / Developer | Total farm capacity = N × Prated |
| Hub Height (H) | meters | Static | OEM / KREDL data | For hub-height wind speed extrapolation |
| Rotor Diameter (D) | meters | Static | OEM datasheet | Swept area A = π(D/2)²; P = 0.5ρAV³Cp |
| Power Curve (V → P) | MW vs m/s | Static | OEM datasheet | Maps wind speed to expected power output |
| Cut-in / Rated / Cut-out Speed | m/s | Static | OEM datasheet | Operational envelope boundaries |
| Latitude / Longitude | decimal degrees | Static | KREDL / site survey | For spatial NWP interpolation, wake modeling |
| Terrain Roughness (z₀) | meters | Static | Site survey / land cover | Log-law wind extrapolation correction |
| Orography / Elevation | m AMSL | Static | SRTM DEM data | Pressure & density correction, flow channeling |
| OPERATIONAL / SCADA DATA |  |  |  |  |
| Active Power Output (P_actual) | MW | 10-min / 1-hr | SCADA / SLDC SRPC portal | Ground truth — primary target variable |
| Capacity Factor (CF) | % (P/Prated) | Hourly | Computed | Normalized performance metric |
| Availability Factor | % | Daily | SCADA / O&M logs | Accounts for planned/unplanned outages |
| Curtailment Flag | 0/1 (boolean) | 10-min | SCADA / SLDC logs | Mark curtailed periods to exclude from training |
| Turbine Status Code | integer code | 10-min | SCADA | Filter fault / maintenance periods |
| Pitch Angle | degrees | 10-min | SCADA | Indicates power curve behavior; early fault detection |
| Rotor Speed (RPM) | RPM | 10-min | SCADA | Correlates with wind speed; drivetrain state |
| Nacelle Wind Speed (V_nac) | m/s | 10-min | SCADA anemometer | Local measured wind (vs NWP); data fusion |
| Grid Injection (MU) | Million Units | Daily / Monthly | SLDC / KPTCL metering | Official generation accounting data |
| TEMPORAL / CALENDAR FEATURES |  |  |  |  |
| Hour of Day (sin/cos encoded) | 0-23 → sin, cos | Hourly | Computed from timestamp | Diurnal wind pattern; avoid discontinuity at midnight |
| Day of Year (sin/cos encoded) | 1-365 → sin, cos | Daily | Computed from timestamp | Seasonal pattern (monsoon, post-monsoon peaks) |
| Month / Season | 1-12 / categorical | Monthly | Computed | Karnataka: Jun-Sep monsoon = peak wind |
| Year (trend feature) | integer | Annual | Computed | Capacity additions, turbine aging trend |
| Public Holidays / Grid Demand | 0/1 | Daily | GoK calendar | Grid curtailment risk during low demand periods |
| FORECAST / NWP MODEL INPUTS |  |  |  |  |
| GFS Wind Speed Forecast (10m, 100m) | m/s | Hourly (6-hr update) | NOAA GFS (free API) | Day-ahead forecasting input |
| ECMWF Wind Forecast (100m) | m/s | Hourly | Copernicus (free tier) | Most accurate global NWP for wind |
| ERA5 Reanalysis (historical) | m/s, hPa, K | Hourly (1979-now) | Copernicus Climate Store | Historical training data for wind & weather |
| MERRA-2 Reanalysis | m/s, Pa, K | Hourly (1980-now) | NASA GES DISC (free) | Alternative reanalysis; bias correction |
| IMD Forecast (Hubli/Bellary) | Wind speed, direction | 3-hourly | IMD API / data.gov.in | Local NWP; India Meteorological Department |
| DERIVED / ENGINEERED FEATURES |  |  |  |  |
| Wind Power Density (WPD) | W/m² | Hourly | Computed: 0.5ρV³ | Energy content of wind per unit area |
| Weibull k & c Parameters | dimensionless, m/s | Monthly | Fitted from wind speed distribution | Statistical characterization of wind regime |
| Rolling Mean Wind Speed (1h, 6h, 24h) | m/s | Hourly | Computed from SCADA/NWP | Trend feature; reduces noise |
| Wind Speed Forecast Error (NWP - actual) | m/s | Hourly | Computed | Bias correction feature; model calibration |
| Wake Loss Factor | % | 10-min | Wake model (Jensen/Fuga) | Inter-turbine energy loss; layout dependent |
| Normalized Power (P/Prated) | 0-1 | 10-min | Computed | Removes capacity differences; model target |

# 3. Core Generation Calculation Formulas

These physics-based equations are essential for data preprocessing, feature engineering, and building physics-informed ML models.

| Calculation | Formula | Parameters & Notes |
| --- | --- | --- |
| Wind Power Output | P = ½ × ρ × A × V³ × Cp | ρ=air density(kg/m³), A=swept area(m²), V=wind speed(m/s), Cp=power coeff(max~0.59) |
| Air Density Correction | ρ = P_atm / (R_specific × T) | P_atm=pressure(Pa), R=287 J/(kg·K), T=temp(K). ρ ≈ 1.225 kg/m³ at STP |
| Hub-Height Wind Extrapolation (Log Law) | V_hub = V_ref × ln(H_hub/z₀) / ln(H_ref/z₀) | z₀=roughness length. For Karnataka farmland z₀ ≈ 0.03-0.1m |
| Hub-Height Wind Extrapolation (Power Law) | V_hub = V_ref × (H_hub/H_ref)^α | α=wind shear exponent ≈ 0.14 (open terrain); 0.20 (mixed) |
| Capacity Factor | CF = E_actual / (P_rated × T_hours) × 100% | E_actual=actual energy(MWh), T_hours=period hours. Karnataka CF ≈ 25-35% |
| Weibull Wind Distribution | f(v) = (k/c) × (v/c)^(k-1) × exp(-(v/c)^k) | k=shape (~1.8-2.2 in Karnataka), c=scale parameter (m/s) |
| Annual Energy Production (AEP) | AEP = ∫ P(v) × f(v) dv × 8760 × N_turbines | Integrate power curve against wind speed distribution |
| Wake Loss (Jensen Model) | ΔV/V₀ = (1-√(1-Ct)) × (D/(D+2kx))² | Ct=thrust coeff, k=wake decay(~0.04 offshore/0.075 onshore), x=distance |
| Turbulence Intensity | TI = σ_V / V_mean × 100% | σ_V=std dev of wind speed over 10-min window. TI<10% is low |
| Normalized Power | P_norm = P_actual / P_rated | 0=offline, 1=full rated output. Use as ML target for multi-plant model |

# 4. Where to Get the Data — Free & Open Sources

| Source | Data Available | URL / Access | Cost |
| --- | --- | --- | --- |
| KREDL Portal | District-wise commissioned wind project list, developer names, capacity | kredl.karnataka.gov.in | Free |
| SLDC Karnataka | 15-min actual generation, scheduling, drawl data per plant/feeder | sldc.karnataka.gov.in | Free |
| KPTCL SCADA / State Load Dispatch | Grid injection metering at 220kV pooling substations | kptcl.com / SLDC | Free (RTI if needed) |
| ERA5 Reanalysis (Copernicus) | Hourly wind at 10m, 100m; temp, pressure 1979-present; 0.25° grid | cds.climate.copernicus.eu | Free (registration) |
| MERRA-2 (NASA) | Hourly wind reanalysis 1980-present; 0.5°×0.625° grid | disc.gsfc.nasa.gov | Free (registration) |
| NOAA GFS Forecast | Hourly global wind forecast; 0.25° grid; updated every 6 hrs | nomads.ncep.noaa.gov | Free |
| IMD Pune / data.gov.in | Surface and upper-air observations; forecast products for Karnataka | data.gov.in/datasets/weather | Free |
| NIWE (National Wind Atlas) | Karnataka wind maps at 80m, 100m, 120m; site-specific wind data | niwe.res.in | Free |
| MNRE / CEA Annual Reports | State-wise annual wind generation (MU), installed capacity updates | mnre.gov.in / cea.nic.in | Free |
| Open-Meteo API | Free hourly historical + forecast wind at 80m/120m; ERA5-based | open-meteo.com | Free (no key needed) |
| Global Wind Atlas 3.0 (DTU/WB) | Mean wind speed, Weibull parameters; 250m resolution; Karnataka maps | globalwindatlas.info | Free |
| IRENA Open Data | Country/state RE generation statistics and capacity factors | irena.org/Statistics | Free |

# 5. Recommended ML Modeling Pipeline

## Step 1 — Data Collection & Alignment

- Download ERA5/MERRA-2 historical wind data (1980-2024) for Karnataka bounding box (14°N–18°N, 74°E–78°E)

- Collect SCADA/SLDC actual generation data for each plant (15-min or 1-hr resolution)

- Obtain static parameters: power curves, hub heights, lat/lon, installed capacity per plant from KREDL / developer data

- Align all data to a common UTC timestamp index; resample to 1-hour resolution

## Step 2 — Preprocessing

- Remove curtailed periods (use SCADA curtailment flag or SLDC dispatch instructions)

- Filter turbine fault/maintenance periods using status codes

- Apply air density correction to wind speed using co-located temperature and pressure

- Extrapolate 10m NWP wind to hub height using power law (α) or log law (z₀)

- Cyclically encode hour and day-of-year using sin/cos transformation

## Step 3 — Feature Engineering

- Compute Wind Power Density (WPD = 0.5 × ρ × V³) as a super-feature

- Add rolling statistics: 1h, 6h, 24h mean/std of wind speed

- Compute NWP forecast error (bias) as a correction feature

- Create plant-level embeddings (or one-hot encode plant ID) for multi-plant models

- Fit Weibull k and c parameters per month/season per plant location

## Step 4 — Model Architecture Options

- Baseline: Gradient Boosting (XGBoost/LightGBM) — fast, interpretable, strong for tabular wind data

- Time-series: LSTM / Temporal Fusion Transformer (TFT) — captures diurnal and seasonal patterns

- Physics-informed: Combine power curve lookup with ML residual correction

- Ensemble: Blend NWP-physics model + ML model for best day-ahead accuracy

## Step 5 — Evaluation Metrics

- RMSE (Root Mean Square Error) — penalizes large errors; primary metric

- MAE (Mean Absolute Error) — robust to outliers

- MAPE (Mean Absolute Percentage Error) — caution: unreliable near zero generation

- Normalized RMSE (nRMSE = RMSE/Prated) — compare across plants of different size

- Skill Score vs. persistence forecast — measures improvement over naive baseline

# 6. Karnataka Wind Seasonality — Key Patterns for Model

| Season | Months | Typical CF | Characteristics |
| --- | --- | --- | --- |
| Southwest Monsoon | June – September | 45-60% | Peak wind season. Strong, consistent westerly/south-westerly winds 6-10 m/s+. Majority of annual generation occurs in this window. |
| Post-Monsoon | October – November | 20-35% | Transitional period. Variable wind direction. Moderate speeds. Grid demand peaks — valuable generation window. |
| Winter | December – February | 15-25% | Relatively calm. Northeast monsoon influence in coastal areas. Cooler temperatures improve air density (slight power boost). |
| Pre-Monsoon / Summer | March – May | 20-30% | Afternoon thermal winds possible. High temperatures reduce air density. Pre-monsoon onset brings increasing wind activity in May. |

Karnataka's Tuppadahalli wind farm (Chitradurga) was recognized as India's best-performing wind farm in FY 2021-22, with a capacity factor significantly exceeding the state average. The Gadag and Vijayapura districts consistently show the highest annual mean wind speeds (7-8 m/s at 100m), making them the model's highest-confidence prediction zones.

Document prepared for Karnataka Wind Generation Forecasting ML Project  |  Data as of May 2026  |  Sources: KREDL, CEA, MNRE, ERA5, NIWE